|  |  |  | INTERIOR DECORATION WORKS FOR UNITED INDIA INSURANCE CO. LTD. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 8th FLOOR , KANCHANJUNGA BUILDING , 18 BARAKHAMBA ROAD, NEW DELHI-110001. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | BRANCH OFFICE AT UDYOG NAGAR METRO STATION , NEW DELHI. |  |  |  |  |  |  |  |  |  |
|  |  |  | ID WORKS |  |  |  |  |  |  |  |  |  |
| S NO |  |  | ITEM / WORKS | SUB | SPECIFICATIONS | UNITS |  | QTY | RATES IN | RATES IN FIGURES | AMOUNT | TOTAL |
|  |  |  |  | HEAD |  |  |  |  | WORDS |  |  |  |
| A |  |  | CIVIL WORK |  |  |  |  |  |  |  |  |  |
|  |  |  | DISMANTLING |  |  |  |  |  |  |  |  |  |
|  |  |  | WORK |  |  |  |  |  |  |  |  |  |
|  | I |  | Disposal of malba |  |  | LS |  | 3.00 |  |  |  |  |
|  |  |  | TOTAL COST OF |  |  |  |  |  |  |  |  |  |
|  |  |  | THE WORKS |  |  |  |  |  |  |  |  |  |
|  |  |  | PROPOSED CIVIL |  |  |  |  |  |  |  |  |  |
| B |  |  | CONSTRUCTIVE |  |  |  |  |  |  |  |  |  |
|  |  |  | WORKS |  |  |  |  |  |  |  |  |  |
|  |  |  | CIVIL WORK |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Providing & laying 75mm thick cement concrete |  |  |  |  |  |  |  |
|  |  |  |  |  | flooring (IPS) in 1:2:4 (1 cement :2 coarse sand: 4 |  |  |  |  |  |  |  |
|  |  |  | Flooring ( Cement |  | graded stone aggrgate 20mm nominal size) finished |  |  |  |  |  |  |  |
|  | I |  |  | i | with floating coat of neat cement including cement | CMT |  | 1.00 |  |  |  |  |
|  |  |  | Concrete) |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | slurry, rounding off edges laid in alternate bays , |  |  |  |  |  |  |  |
|  |  |  |  |  | including mixing of the cement admixture to enhance |  |  |  |  |  |  |  |
|  |  |  |  |  | the impervious quality of the cc. complete |  |  |  |  |  |  |  |
|  | II |  | Plaster |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | P / A 12mm thick plaster in cement mortar 1 :4 to all |  |  |  |  |  |  |  |
|  |  |  |  |  | types of RCC / brick work, columns, surfaces including |  |  |  |  |  |  |  |
|  | b |  | 1:4 Cement Mortar |  | wall tops, jambs and sills etc. in line level and plumb, | SMT |  | 2.00 |  |  |  |  |
|  |  |  |  |  | including smooth cement finish, including providing |  |  |  |  |  |  |  |
|  |  |  |  |  | necessary grooves, if required |  |  |  |  |  |  |  |
|  |  |  | TOTAL COST OF |  |  |  |  |  |  |  |  |  |
|  |  |  | THE WORKS |  |  |  |  |  |  |  |  |  |
| C |  |  | FLOORING |  |  |  |  |  |  |  |  |  |

AV ARCHITECTS	Page 1 of 6	BRANCH OFFICE	UDYOG NAGAR METRO STATION, NEW DELHI.

S NO

ITEM / WORKS

SUB

HEAD

SPECIFICATIONS

UNITS

QTY

RATES IN

WORDS

RATES IN FIGURES

AMOUNT

TOTAL

|  |  |  |  | P / L First class 600 x 600mm vitrified tiles brand asian |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | granito model Morocco using 1:4 cement mortar in |  |  |  |  |  |  |
|  |  |  |  | floor & 100mm high dado, with machine cutting as |  |  |  |  |  |  |
|  |  |  |  | required, gaps treated with grout with an admixture of |  |  |  |  |  |  |
|  |  |  |  | pigment to match the shade of tile and joints to match |  |  |  |  |  |  |
|  |  |  |  | the ones on floor and with dado, as per drawing and |  |  |  |  |  |  |
|  | I | Vitrified Tile Flooring | i | specifications including providing & APPLYING POP | SMT | 85.00 |  |  |  |  |
|  |  |  |  | ON FLOORING FOR FLOOR PROTECTION WITH |  |  |  |  |  |  |
|  |  |  |  | PLASTIC SHEET OF 150 TO 200 MICRONS |  |  |  |  |  |  |
|  |  |  |  | THK.BETWEEN THE FLOORING AND POP. THE |  |  |  |  |  |  |
|  |  |  |  | RATE MUST INCLUDE THE CLEANING OF THE POP |  |  |  |  |  |  |
|  |  |  |  | AND PLASTIC ON COMPLETION OF OTHER |  |  |  |  |  |  |
|  |  |  |  | WORKS TO EXPOSE THE FLOORING etc. |  |  |  |  |  |  |
|  |  | TOTAL COST OF |  |  |  |  |  |  |  |  |
|  |  | THE WORKS |  |  |  |  |  |  |  |  |
| E |  | FALSE CEILING |  |  |  |  |  |  |  |  |
|  |  | WORKS |  |  |  |  |  |  |  |  |
|  |  |  |  | P/Fixing of Mineral fiber ceiling system with mineral |  |  |  |  |  |  |
|  |  |  |  | fibre tiles having a life long guarantee with Humidity |  |  |  |  |  |  |
|  |  |  |  | resistance of 95%, NRC of 0.55- 0.6,Sound |  |  |  |  |  |  |
|  | I | T GRID CEILLING |  | Attenuation -34db,in module of size of 600mm | SMT | 80.00 |  |  |  |  |
|  |  | ARMSTRONG |  | x600mm x15mm,micro look edges to be laid on |  |  |  |  |  |  |
|  |  |  |  | Silouhette Grid.. The cost would include cost for |  |  |  |  |  |  |
|  |  |  |  | suspenders, dash-fasterners, associated hardware, |  |  |  |  |  |  |
|  |  |  |  | cutting for lights & speakers in the ceiling tile). |  |  |  |  |  |  |
|  | II | POP PUNNING ON |  |  | SMT | 15.00 |  |  |  |  |
|  |  | WALL |  |  |  |  |  |  |  |  |
|  |  | TOTAL COST OF |  |  |  |  |  |  |  |  |
|  |  | THE WORKS |  |  |  |  |  |  |  |  |
| G |  | DOOR WORKS |  | Note 1: All doors to include door acccessories- Door |  |  |  |  |  |  |
|  |  | (Shutter ) |  | closer, door handle, door lock, and door buffer. |  |  |  |  |  |  |
|  |  |  |  | Note 2: All the floor springs/ door closers to be Used |  |  |  |  |  |  |
|  |  |  |  | from Dorset make. Door closer/ floor spring to be |  |  |  |  |  |  |
|  |  |  |  | selected as per the weight of the door, as specified by |  |  |  |  |  |  |
|  |  |  |  | the manufacturer. |  |  |  |  |  |  |
|  |  |  |  | Note 3: All door Handles and mortic Locks to be from |  |  |  |  |  |  |
|  |  |  |  | Dorset or Ozone. Door Handle for toughened glass |  |  |  |  |  |  |
|  |  |  |  | doors- SD 10 P SS- 12" long handle SS Handle from |  |  |  |  |  |  |
|  |  |  |  | Dorset. All patch fittings to be from Ozone make. |  |  |  |  |  |  |
|  |  |  |  | Note 4: All hinges being used to be in SS and all |  |  |  |  |  |  |
|  |  |  |  | hinges to be Ball Bearing hinges , Hinges (4") to be |  |  |  |  |  |  |
|  |  |  |  | used per door. |  |  |  |  |  |  |

AV ARCHITECTS	Page 2 of 6	BRANCH OFFICE	UDYOG NAGAR METRO STATION, NEW DELHI.

S NO

ITEM / WORKS

SUB

HEAD

SPECIFICATIONS

UNITS

QTY

RATES IN

WORDS

RATES IN FIGURES

AMOUNT

TOTAL

Note 5: All door chowkhats to be in first class Steem

beach, unless specified otherwise.

|  | Glazed Door shutter | Providing and fixing Glass door ( 6 mm thick) with |  |  |
| --- | --- | --- | --- | --- |
|  | including harware and |  |  |  |
| I |  | Lower Stile of 8" and upper stile of 5" complete | SMT | 5.80 |
|  | door closures and |  |  |  |
|  |  | inclusive all fixing accessories and all incidentals. |  |  |
|  | fittings |  |  |  |
|  |  | Providing & fixing double shutter TOUGHENED Glass |  |  |
|  |  | doors of size as per drawing fabricated with 12mm thk |  |  |
|  |  | TOUGHENED glass (Modi Guard / Saint Gobain),with |  |  |
|  |  | approved patch fittings as shown in the standard |  |  |
|  |  | detailed drawing & floor springs. It shall have approved |  |  |
| II | Front Glass Shutter | stainless steel door handles on both sides and suitable | SMT | 7.00 |
|  |  | locking arrangement etc. all as directed &to the entire |  |  |
|  |  | satisfaction of Architect / owner. |  |  |
|  |  | All the fittings shall be of Ozone make, Handle of Neki |  |  |
|  |  | make - H shape (25mm dia - 2' long) |  |  |
| III | Architrave Moulding | Providing and fixing Architrave Moulding | RMT | 31.00 |
| IV | Door Frames | Providind and Fixing Woodwork in Chowkhats Steem | RMT | 15.70 |
|  |  | Beech wood unless specified. |  |  |
|  | TOTAL COST OF |  |  |  |
|  | THE WORKS |  |  |  |
|  | PARTITION AND | Note 1: All the wooden members/ boards/ panels being |  |  |
| I |  | used will have to be given anti-termite ( with 5 years |  |  |
|  | PANELLING WORKS |  |  |  |
|  |  | guarantee) and fire retardant treatment at site. |  |  |
|  |  | Note 2: All the laminates to be used from "Woodgrain" |  |  |
|  |  | series from Merino lam or equivalent from Century/ |  |  |
|  |  | Greenlam/ Royal Touch, min. 1mm thk conforming to |  |  |
|  |  | IS 2217. |  |  |
|  |  | Note 3: All the glass to be float glass (unless specified |  |  |
|  |  | otherwise) from ASAHI / SAINT GOBAIN. All patch |  |  |
|  |  | fittings to be from Ozone make. |  |  |
|  |  | Note 4: All Gypsum Board (MR Grade) and GI sections |  |  |
|  |  | to be of Gypsum India make. |  |  |

AV ARCHITECTS	Page 3 of 6	BRANCH OFFICE	UDYOG NAGAR METRO STATION, NEW DELHI.

S NO

ITEM / WORKS

SUB

HEAD

SPECIFICATIONS

UNITS

QTY

RATES IN

WORDS

RATES IN FIGURES

AMOUNT

TOTAL

|  |  |  |  | P/F glazed partition with following details: .both side |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FULL HEIGHT HALF |  | 12mm th. comm. ply, Bottom to have laminated skirting |  |  |  |  |  |  |
|  |  |  |  | of approx. 100mm width. complete as per detailed |  |  |  |  |  |  |
|  | i | TRANSPARENT |  |  | SMT | 45.00 |  |  |  |  |
|  |  |  |  | dwgs. Note (1) cost not to include the polishing/ |  |  |  |  |  |  |
|  |  | PARTITION |  |  |  |  |  |  |  |  |
|  |  |  |  | staining of wood as per approved sample , (2) Cost to |  |  |  |  |  |  |
|  |  |  |  | include float glass of 6mm thk. |  |  |  |  |  |  |
|  |  | LOW HEIGHT |  |  |  |  |  |  |  |  |
|  | ii | PARTITION WITH |  | Top rail 3"X1" steam beach wood | SMT | 20.00 |  |  |  |  |
|  |  | TOP RAIL |  |  |  |  |  |  |  |  |
|  | iii | WALL PANELLING |  | P/F with wooden frame and 12mm th. Ply | SMT | 4.00 |  |  |  |  |
|  |  | WORK |  |  |  |  |  |  |  |  |
|  |  | GLASS FOR LOW |  | Providing & laying 12mm TOUGHENED GLASS |  |  |  |  |  |  |
|  | iv |  |  | EDGE POLISHING ALL SIDE FIXING WITH SS | SMT | 2.00 |  |  |  |  |
|  |  | HEIGHT PARTITION |  |  |  |  |  |  |  |  |
|  |  |  |  | CLAMP. |  |  |  |  |  |  |
|  |  | TOTAL COST OF |  |  |  |  |  |  |  |  |
|  |  | THE WORKS |  |  |  |  |  |  |  |  |
| J |  | PAINT AND |  |  |  |  |  |  |  |  |
|  |  | POLISHING |  |  |  |  |  |  |  |  |
|  |  |  |  | Providing and applying plastic emulsion of approved |  |  |  |  |  |  |
|  |  |  |  | colour & shade, cleaning the surface and make it free |  |  |  |  |  |  |
|  |  |  |  | from dust by scrapping, cleaning etc., including |  |  |  |  |  |  |
|  |  |  |  | application of two coats acrylic putty of approved make |  |  |  |  |  |  |
|  |  |  |  | over the walls and ceiling to make it the same in line |  |  |  |  |  |  |
|  |  |  |  | and level; application of primer coat over the same; |  |  |  |  |  |  |
|  | I | Plastic Emulsion |  | application of minimum two or more coats of plastic | SMT | 180.00 |  |  |  |  |
|  |  |  |  | emulsion of Asian paints / ICI Paints or equivalent upto |  |  |  |  |  |  |
|  |  |  |  | the desired finish of the EIC and protecting till the |  |  |  |  |  |  |
|  |  |  |  | handing over of site including required scafolding, |  |  |  |  |  |  |
|  |  |  |  | cleaning, filling up cracks with approved crack filler etc. |  |  |  |  |  |  |
|  |  |  |  | all complete at all levels and leads to entire satisfaction |  |  |  |  |  |  |
|  |  |  |  | of the Architect/Owner. |  |  |  |  |  |  |
|  | II | MELAMINE POLISH |  | Providing and appliying polish wih melamine coating on | SMT | 9.00 |  |  |  |  |
|  |  |  |  | all exposed wood |  |  |  |  |  |  |
|  |  | TOTAL COST OF |  |  |  |  |  |  |  |  |
|  |  | THE WORKS |  |  |  |  |  |  |  |  |

AV ARCHITECTS	Page 4 of 6	BRANCH OFFICE	UDYOG NAGAR METRO STATION, NEW DELHI.

| S NO |  | ITEM / WORKS | SUB |  | SPECIFICATIONS | UNITS | QTY | RATES IN | RATES IN FIGURES | AMOUNT | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | HEAD |  |  |  |  | WORDS |  |  |  |
| H |  | FURNITURE WORKS |  |  |  |  |  |  |  |  |  |
|  | 3 | TABLE |  | Scope : Providing , Manufacturing, Fixing in |  |  |  |  |  |  |  |
|  |  |  |  | Position |  |  |  |  |  |  |  |
|  |  |  |  | The top : 19 mm thk Commercial board with 1.0 |  |  |  |  |  |  |  |
|  |  |  |  | mm laminate in the pattern approved. The top |  |  |  |  |  |  |  |
|  |  |  |  | with TW Cornice moulding 2 1/4" x 1 1/2" all |  |  |  |  |  |  |  |
|  |  |  |  | arround. |  |  |  |  |  |  |  |
|  |  |  |  | Facia: 19 mm thk Commercial board with 1.0 mm |  |  |  |  |  |  |  |
|  |  |  |  | laminate | top in the pattern approved. |  |  |  |  |  |  |
|  |  |  |  | Structure paint :with fire paint of Vampire or |  |  |  |  |  |  |  |
|  |  |  |  | equivalent |  |  |  |  |  |  |  |
|  |  |  |  | Finishes: The Steam beech wooden surfaces to |  |  |  |  |  |  |  |
|  |  |  |  | be melamnine polished. |  |  |  |  |  |  |  |
|  | a | MGR |  | Size: 7'0" | x 3'0" with side unit 3' x 18" | NOS | 1.00 |  |  |  |  |
|  | b | ASST/MGR |  | Size: 5'0" | x 2'0" with side unit 3' x 18" | NOS | 2.00 |  |  |  |  |
|  | 5 | LOW HEIGHT |  |  |  | SMT | 6.00 |  |  |  |  |
|  |  | CABINET |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Scope : Providing , Manufacturing, Fixing in |  |  |  |  |  |  |  |
|  |  |  |  | Position |  |  |  |  |  |  |  |
|  |  |  |  | Height: 30" high |  |  |  |  |  |  |  |
|  |  |  |  | Depth : 18" |  |  |  |  |  |  |  |
|  |  |  |  | The top : 25 mm thk Post formed top Bull |  |  |  |  |  |  |  |
|  |  |  |  | Nose/edge taping |  |  |  |  |  |  |  |
|  |  |  |  | Front Shutter: 19 mm thk Post formed top , edge |  |  |  |  |  |  |  |
|  |  |  |  | taping |  |  |  |  |  |  |  |
|  |  |  |  | The Box Structure: 19 mm thk PLPB of approved |  |  |  |  |  |  |  |
|  |  |  |  | laminate shade. |  |  |  |  |  |  |  |
|  |  |  |  | Fixing : Fixed to the exsiting Partition with |  |  |  |  |  |  |  |
|  |  |  |  | Fasteners and Gitti |  |  |  |  |  |  |  |
|  |  |  |  | Other: Includes all hardware for fixing in plumb |  |  |  |  |  |  |  |
|  |  |  |  | line, Adhesive of Pidilite, including Hardware - 3" |  |  |  |  |  |  |  |
|  |  |  |  | to 4" SS handles of approved design, self closing |  |  |  |  |  |  |  |
|  |  |  |  | 0 to 8 crenk hinges, Ball catcher, Chitakni, and |  |  |  |  |  |  |  |
|  |  |  |  | Lock by Godrej. |  |  |  |  |  |  |  |
|  |  |  |  | Method of meaurement: Front side |  |  |  |  |  |  |  |

AV ARCHITECTS	Page 5 of 6	BRANCH OFFICE	UDYOG NAGAR METRO STATION, NEW DELHI.

| S NO |  | ITEM / WORKS | SUB | SPECIFICATIONS | UNITS | QTY | RATES IN | RATES IN FIGURES | AMOUNT | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | HEAD |  |  |  | WORDS |  |  |  |
|  |  |  |  | Providing and fixing in position 25mm thk Postformed |  |  |  |  |  |  |
|  |  |  |  | working table of approved laminate finish. Including |  |  |  |  |  |  |
|  | 6 | Post form Work top |  | Manufacturing /Providing / Fixing Top & structure made | SFT | 63 |  |  |  |  |
|  |  |  |  | of 19mm thk prelam board edge binded.as per drawing |  |  |  |  |  |  |
|  |  |  |  | etc |  |  |  |  |  |  |
|  |  |  |  | Cabinets having full laminated PLPB shutters 3/4" |  |  |  |  |  |  |
|  |  |  |  | thick, with drawer units and a shutter unit out of 3.4" |  |  |  |  |  |  |
|  |  |  |  | thick PLB @1'4" -1'6" ht (refer drawing), with shutters |  |  |  |  |  |  |
|  | 7 | Pedestal Units |  | out of postformed PLPB panels, top to be out of | NOS | 7 |  |  |  |  |
|  |  |  |  | postformed PLPB 19 mm, the sides to be 3/4" PLPB, |  |  |  |  |  |  |
|  |  |  |  | laminated with edge lipping, complete as per drawing, |  |  |  |  |  |  |
|  |  |  |  | 2'6 " ' high, 1' 8" deep. |  |  |  |  |  |  |
|  | 8 | Key board ms powder |  |  | NOS | 12 |  |  |  |  |
|  |  | coating |  |  |  |  |  |  |  |  |
|  |  | TOTAL COST OF |  |  |  |  |  |  |  |  |
|  |  | THE WORKS |  |  |  |  |  |  |  |  |
|  |  | TOTAL OF ALL |  |  |  |  |  |  |  |  |
|  |  | THE WORKS |  |  |  |  |  |  |  |  |

AV ARCHITECTS	Page 6 of 6	BRANCH OFFICE	UDYOG NAGAR METRO STATION, NEW DELHI.